DIGITAL SIGNAL PROCESSING

CMPE 476

HW 3

Solve All Questions.

Q1. Let x(n) be a complex values sequence with real part xR(n) and the imaginary part xI(n). Prove the following z-transform two relations:

Must you use only MATLAB in your proof, and for x(n), use two random sequences for real and the imaginary parts.

Code

Output

Q2. The z-transform of x(n) is X(z) = 1/(1 - 0.5z−1), |z| ≥ 0.5. Determine the z-transforms of the following sequences and indicate their region of convergence:

x2(n) = (1+n) x(n)

Q3. Determine the following inverse z-transforms using the partial fraction expansion method.

X1(z) = z-1 / [1 – 0.25z-1 – 0.15z-2]

- Get x(n) analytically

- Proof that using MATLAB

Code

Output

Q4. Solve the following difference equation for y(n) using the one-sided z-transform approach.

y(n) = 0.5 y(n-2) + x(n) – x(n-1), n ≥ 0; y(-1) = Y(-2) = 1

x(n) = (0.5)n u(n)

Generate the first 20 samples of y(n) using MATLAB and compare them with your answer

Code

Output

Q5. For the linear, causal, and time-invariant systems described by the following difference equation:

y(n) = 2x(n) + 0.9y(n − 1)

determine (i) the impulse response representation, (ii) the system function representation, (iii) the pole-zero plot, and (iv) the output y(n) if the input is

x(n) = u(n-1).